# Self Diagnosis & Protection

- When A/C begins protection situation, it will continue running. Run indicator will be lighting;

2. When A/C have abnormal problem, it will stop running. Run indicator will be blinking;

3. Every abmormal problem or normal protection, it will indicate twice. If there is more than two problems or protections, they will indicate one by one in three seconds;

4. After A/C stops running due to abnormal problem, please press remote controller to set A/C off;

5. Diagnosis of  abnormal problem and normal protection ：

| Abnormal problem & Protection | Indicator |  |  | ## Remark |
| --- | --- | --- | --- | --- |
|  | Run indicator（left） | Sleep indicator（middle） | Time indicator （right） | - lighting；○off； ☆blinking |
| Abnormity to the pipe sensor of indoor unit | ☆ | ☆ | ☆ | Belonging to abnormal problem，A/C stops running， Repair of service is necessary then. |
| Abnormity to the room temperature sensor of indoor unit | ☆ | ☆ | ● |  |
| Abnormity to the pipe sensor of outdoor unit | ☆ | ☆ | ○ |  |
| Abnormity to outdoor unit（lack of refrigerent or too low temprerature of outdoor situation） | ☆ | ○ | ☆ |  |
| Frozen protection | ○ | ☆ | ☆ | Normal protection，A/C will recover itself and run again，no need to repair |
| Overload protection | ○ | ☆ | ● |  |
| Get rid of frost | ○ | ☆ | ○ |  |
| Cool air protection | ○ | ● | ○ |  |